# LABORATORY 1

## PROGRAMMING BASED ON DATA TRANSFER AND ARITHMETIC OPERATIONS

OBJECTIVE

To familiar with the use of data transfer and arithmetic instruction set of 8085

PRELAB

Read data transfer  and arithmetic instructions from textbook in detail.

OVERVIEW OF DATA TRANSFER INSTRUCTIONS

| Opcode | Operand | Description |
| --- | --- | --- |
| MOV | Rd, Rs | Copies data from source resister Rs to Rd |
| MVI | R, 8-bit | Copies 8-bit data to resister R |
| LXI | Rp, 16-bit | Copies 16-bit data to register pair Rp |
| LXI | SP, 16-bit | Copies 16-bit data to stack pointer SP |
| MOV | R,M | Copies 8-bit data stored at memory location specified by the content of HL register to register R |
| LDAX | B/D | Copies 8-bit data stored at memory location specified by the content of BC or DE register to the accumulator |
| LDA | 16-bit | Copies 8-bit data stored at memory location specified by 16-bit to the accumulator |
| MOV | M,R | Copies data from a register R, into the memory location specified by the HL pair |
| STAX | B/D | Stores the content of the accumulator to the memory location specified by the BC or DE pair |
| STA | 16-bit | Store the content of the accumulator to memory address |
| MVI | M, 8-bit | Store 8-bit data to the memory location specified by the register pair HL |
| XCHG |  | Exchange H and L with D and E respectively |

| ADD | R | Adds the content of register R to the content of the accumulator |
| --- | --- | --- |
| ADI | 8-bit | Adds the 8-bit Data to the content of the accumulator |
| SUB | R | Subtracts content of register R from the content of the accumulator |
| SUI | 8-bit | Subtracts 8-bit Data from the content of the accumulator |
| INR | R | Increases the content of register R by 1 |
| DCR | R | Decreases the content of register R by 1 |
| ADD | M | Adds the content of memory location specified by register pair HL to the content of the accumulator |
| SUB | M | Subtracts the content of memory location specified by register pair HL from the content of the accumulator |
| INR | M | Increment the content of the memory location specified by register pair HL by 1. |
| DCR | M | Decrement the content of the memory location specified by register pair HL by 1. |

EXERCISES

- Load the memory address 2100H with 0x45 and transfer it to accumulator. Copy accumulator to register B, C, D, E, H and L.

- Load the accumulator with 0x45 and save accumulator at memory location 2105H.

- Write instructions to load 0x65 in register C and 0x92 in the accumulator. Store the content of both register at memory location 2122H and 2212H.

- Write a program to exchange the content of B and C register.

- Write a program to exchange the content of memory location 2000H and 2001H.