LABORATORY 3

### PROGRAMMING BASED ON BRANCH OPERATIONS

OBJECTIVE

To familiar with the use of branch operation related instructions in 8085 programming

PRELAB

Read and understand utilization of the instructions related to branch operations.

OVERVIEW OF BRANCH INSTRUCTIONS

| Opcode | Operand | Description |
| --- | --- | --- |
| JMP | 16-bit | Jump on the 16-bit memory address |
| JC | 16-bit | Jump on Carry |
| JNC | 16-bit | Jump on no Carry |
| JZ | 16-bit | Jump on Zero |
| JNZ | 16-bit | Jump on no Zero |
| JP | 16-bit | Jump on Plus |
| JM | 16-bit | Jump on Minus |
| JPE | 16-bit | Jump on Even Parity |
| JPO | 16-bit | Jump on Odd Parity |

EXERCISES

- Write a program to multiply two 8 bit data stored at location 2101H and 2102H.

Ans. LDA 2101H

MOV D,A

LDA 2102H

MOV E,A

MVI A,00H

ABC: ADD E

JNC BCD

INR B

BCD: DCR D

JNZ ABC

HLT

- Write a program to add an array of 10 bytes data stored from location 5000H to 5009H and store the result at 500AH and 500BH.

Ans. LXI H,5009H

MOV D,L

INR D

MVI A,00H

ABC: MOV C,M

DCR L

ADD C

JNC CDE

INR B

CDE: DCR D

JNZ ABC

MOV H,A

MOV L,B

SHLD 500AH

HLT

- Write a program to subtract an array of 10 bytes data stored from location 5000H to 5009H and store the result at 500AH and 500BH.

Ans. LXI H,5009H

MOV D,L

INR D

MOV A,M

ABC:DCR L

MOV C,M

SBB C

JNC CDE

INR B

CDE: DCR D

JNZ ABC

MOV H,A

MOV L,B

SHLD 500AH

HLT

- Write a program to find largest and smallest numbers in block of array from 2100H to 2109H. Store the result at location 210AH and 210BH.

Ans. LDA 2109H

STA 210BH

LXI H,2109H

MOV D,L

REP: DCR L

CMP M

JNC SML

MOV B,A

LDA 210BH

CMP M

JC BIG

MOV A,B

DCR D

JNZ REP

JZ END

SML: MOV A,M

STA 210AH

DCR D

JNZ REP

BIG: MOV A,M

STA 210BH

MOV A,B

DCR D

JNZ REP

END:HLT

- Write an ALP to sort an array of 10 elements from 2100H in ascending order.

Ans. MVI E,09H

RP0: MVI D,09H

LXI H,2100H

REP: MOV A,M

INR L

CMP M

JNC SML

DCR D

JNZ REP

JMP END

SML: MOV B,M

MOV M,A

DCR L

MOV M,B

INR L

DCR D

JNZ REP

END: DCR E

JNZ RP0

HLT

- Write an ALP to sort an array of 10 elements from 2100H in descending order.

Ans. MVI E,09H

RP0: MVI D,09H

LXI H,2100H

REP: MOV A,M

INR L

CMP M

JC BIG

DCR D

JNZ REP

JMP END

BIG: MOV B,M

MOV M,A

DCR L

MOV M,B

INR L

DCR D

JNZ REP

END: DCR E

JNZ RP0

HLT