Laboratory 5

Programming based on block data transfer

Objective

To familiar with the use of data transfer instruction set of 8086

Prelab

Read data transfer instructions and segment registers from textbook in detail.

Data Transfer instructions

| Instructions | Operands | Description |
| --- | --- | --- |
| MOV | REG, memory memory, REG REG, REG memory, immediate REG, immediate SREG, memory memory, SREG REG, SREG SREG, REG | Copy operand2 to operand1. The MOV instruction cannot: - Set the value of the CS and IP registers. - Copy value of one segment register to another segment register (should copy to general register first). - Copy immediate value to segment register (should copy to general register first). Algorithm: operand1 = operand2 Ex: Mov AX,BX ;Copy contents of BX to AX Mov si,00h ;load Si with 00h |
| PUSH | REG SREG memory | Store 16 bit value in the stack. Algorithm: - SP = SP - 2 - SS:[SP] (top of the stack) = operand |
| POP | REG SREG memory | Get 16 bit value from the stack. Algorithm: Operand = SS : [SP](top of stack) SP = Sp + 2. |
| XCHG | REG, memory memory, REG REG, REG | Exchange values of two operands. Algorithm: operand1 < - > operand2 |
| IN | AL, im.byte AL, DX AX, im.byte AX, DX | Input from port into AL or AX. Second operand is a port number. If required to access port number over 255 - DX register should be used. |
| OUT | AL, ibyte AL, DX AX, DX | Output from AL or AX to port. First operand is a port number. If required to access port number over 255 - DX register should be used. |
| LEA | REG, memory | Load Effective Address. Algorithm: REG = address of memory (offset) |
| XLAT | No Operands | Translate byte from table. Copy value of memory byte at DS:[BX + unsigned AL] to AL register. Algorithm: AL = DS:[BX + unsigned AL] |

Exercises

5.1  An ALP to transfer a given block of data from source memory block to the destination memory block without overlap.

Ans. DATA SEGMENT

BLOCK1 DB 23H,56H,54H,87H,34H,74H

BLOCK2 DB ?

DATA ENDS

CODE SEGMENT

ASSUME CS:CODE,DS:DATA

START:  MOV AX,DATA

MOV DS,AX

LEA SI,BLOCK1

LEA DI,BLOCK2

MOV CL,06

BACK:   MOV AL,[SI]

MOV [DI],AL

INC DI

INC SI

DEC CL

JNZ BACK

INT 21H

CODE ENDS

END START

5.2  An ALP to exchange a block of data located from source to the destination memory locations.

Ans. DATA SEGMENT

BLOCK1 DB 32H,34H,24H,56H,63H,58H

BLOCK2 DB 10H,20H,30H,40H,50H,60H

DATA ENDS

CODE SEGMENT

ASSUME CS:CODE,DS:DATA

START:  MOV AX,DATA

MOV DS,AX

LEA SI,BLOCK1

LEA DI,BLOCK2

MOV CL,06

BACK:	MOV AL,[SI]

MOV BL,[DI]

MOV [DI],AL

MOV [SI],BL

INC SI

INC DI

DEC CL

JNZ BACK

INT 21H

CODE ENDS

END START

5.3  An ALP to transfer a given block of data (word) from source memory block to the destination memory block without overlap.

Ans. DATA SEGMENT

BLOCK1 DW 2452H,7532H,3967H,9675H,3296H,3565H

BLOCK2 DW ?

DATA ENDS

CODE SEGMENT

ASSUME CS:CODE,DS:DATA

START:  MOV AX,DATA

MOV DS,AX

LEA SI,BLOCK1

LEA DI,BLOCK2

MOV CL,06H

BACK:   MOV AX,[SI]

MOV [DI],AX

INC DI

INC DI

INC SI

INC SI

DEC CL

JNZ BACK

INT 21H

CODE ENDS

END START

5.2An ALP to exchange a block of data (word) located from source to the destination memory locations.

Ans. DATA SEGMENT

BLOCK1 DW 2452H,7532H,3967H,9675H,3296H,3565H

BLOCK2 DW 1234H,5678H,9012H,3456H,7890H,1234H

DATA ENDS

CODE SEGMENT

ASSUME CS:CODE,DS:DATA

START:  MOV AX,DATA

MOV DS,AX

LEA SI,BLOCK1

LEA DI,BLOCK2

MOV CL,06H

BACK:   MOV AX,[SI]

MOV BX,[DI]

MOV [DI],AX

MOV [SI],BX

INC DI

INC DI

INC SI

INC SI

DEC CL

JNZ BACK

INT 21H

CODE ENDS

END START