Laboratory 9

To Study the Dos Interrupt and Write a Program to Display a String on Console using DOS Interrupt.

Objective

To learn DOS Interrupt commands.

Prelab

Read Interrupts.

Overview of Dos Interrupts

Interrupt INT 21h:

INT 21h calls DOS functions.

Function 01h - Read character from standard input, result is stored in AL. If there is no character in the keyboard buffer, the function waits until any key is pressed.

Invoked by: AH = 01h

Returns:      AL = character entered.

Example:

Mov AH, 01h

INT 21h

Function 02h - Write a character to standard output.

Invoked by: DL = character to write.

AH =02h

After execution AL = DL.

Example:

Mov AH, 02h

Mov DL, ’a’	; Character to be displayed on screen must be stored in DL reg.

INT 21h

Function 06h – Direct console for input/output. If DL = 0FFH on entry, then this function reads the console. If DL = ASCII character, then this function displays the ASCII character on the console video screen.

Invoked by: Parameters for O/P: DL = 0…255

Parameters for I/P: DL = 255.

Returns: for O/P: AL = DL.

For I/P:  ZF set if no character available & AL = 0

ZF clear if character available & AL = character.

Example:

Mov AH, 06h

Mov DL, 0ffh

INT 21h

Function 09h - Write a string to standard output at DS: DX.

String must be terminated by '$'. The string can be of any length and may contain control characters such as carriage return (0DH) and line feed (0AH).

Invoked by: DS = string to write.

AH = 09h

Example:

Mov AH, 09h

Mov DX,offset str	;Address of the string to be displayed

INT21h

Function 2Ch - Get system time.

Invoked by: AH =2Ch
Return: CH = hour. CL = minute. DH = second. DL = 1/100 seconds.

Example:

Mov AH, 2ch

INT 21h

Function 4Ch – Terminate a process.

Invoked by: AH = 4ch

Return: returns control to the operating system.

Example:

Mov AH, 4Ch

INT 21h

Interrupt INT 10h:

INT 10h calls the BIOS functions. This interrupt often called the video services interrupt as it directly controls the video display in a system.

Function 02h - Set cursor position.

Invoked by: DH = row; DL = column; BH = page number (0...7); AH=02h.

Example:

MOV AH, 02h

MOV BH, 00

MOV DH, 06

MOV DL, 10

INT 10h

Function 03h – Get cursor position.

Invoked by: BH = page number. (In general 0)

AH = 03h

Return: DH = row number; DL = column number; CH = cursor start line;

CL = cursor bottom line.

Example:

Mov BH, 0

Mov AH, 03h

INT 10h

Function 06h – Scroll up window

Invoked by: AL = number of lines by which to scroll. (00h = clear the entire screen.)

BH = attribute used to write blank lines at bottom of window.
        CH, CL = row, column of window's upper left corner.
        DH, DL = row, column of window's lower right corner.

Exercises

9.1 Display string on console using DOS interrupts

Ans.

DATA SEGMENT

STR DB "HELLOTHERE$"

DATA ENDS

CODE SEGMENT

ASSUME CS:CODE,DS:DATA

START:  MOV AX,DATA

MOV DS,AX

MOV AH,09H

LEA DX,STR

INT 21H

MOV AH,4CH

INT 21H

CODE ENDS

END START