Laboratory 6

Programming based on Sorting of An Array of Numbers

Objective

To learn to sort the given data in to ascending & descending order.

Prelab

Read instructions related to Branch and Subroutine Call.

Instruction

| Instructions | Operands | Description |
| --- | --- | --- |
| CALL | procedure name label | Transfers control to procedure, return address is (IP) pushed to stack. |
| RET | No operands Or even immediate date | Return from near procedure. Algorithm: - Pop from stack: - IP if immediate operand is present: SP = SP + operand |

Exercises

6.1  Write an ALP to find the largest number from an array.

Ans. DATA SEGMENT

BLOCK1 DB 21H,87H,42H,47H,24H,82H

DATA ENDS

CODE SEGMENT

ASSUME CS:CODE,DS:DATA

START:  MOV AX,DATA

MOV DS,AX

MOV DL,05

LEA SI,BLOCK1

MOV CL,[SI]

BCD:      INC SI

MOV BL,[SI]

CMP CL,BL

JNC ABC

MOV CL,BL

ABC:      DEC DL

JNZ BCD

INT 21H

CODE ENDS

END START

6.2  Write an ALP to find the smallest number from an array.

Ans. DATA SEGMENT

BLOCK1 DB 21H,87H,42H,47H,04H,82H

DATA ENDS

CODE SEGMENT

ASSUME CS:CODE,DS:DATA

START:  MOV AX,DATA

MOV DS,AX

MOV DL,05

LEA SI,BLOCK1

MOV CL,[SI]

BCD:      INC SI

MOV BL,[SI]

CMP CL,BL

JC ABC

MOV CL,BL

ABC:      DEC DL

JNZ BCD

INT 21H

CODE ENDS

END START

6.3  Write an ALP to sort the given array of numbers in ascending order.

Ans. DATA SEGMENT

BLOCK1 DB 21H,87H,42H,47H,04H,82H

DATA ENDS

CODE SEGMENT

ASSUME CS:CODE,DS:DATA

START:  MOV AX,DATA

MOV DS,AX

MOV DH,05

ABC:       MOV DL,05

LEA SI,BLOCK1

BCD:       MOV CL,[SI]

INC SI

MOV BL,[SI]

CMP CL,BL

JC CDE

MOV [SI],CL

DEC SI

MOV [SI],BL

INC SI

CDE:       DEC DL

JNZ BCD

DEC DH

JNZ ABC

INT 21H

CODE ENDS

END START

6.4  Write an ALP to sort the given array of numbers in descending order.

Ans. DATA SEGMENT

BLOCK1 DB 21H,87H,42H,47H,04H,82H

DATA ENDS

CODE SEGMENT

ASSUME CS:CODE,DS:DATA

START:  MOV AX,DATA

MOV DS,AX

MOV DH,05

ABC:       MOV DL,05

LEA SI,BLOCK1

BCD:       MOV CL,[SI]

INC SI

MOV BL,[SI]

CMP CL,BL

JNC CDE

MOV [SI],CL

DEC SI

MOV [SI],BL

INC SI

CDE:       DEC DL

JNZ BCD

DEC DH

JNZ ABC

INT 21H

CODE ENDS

END START