Laboratory 7

Programming based on Bit Manipulation.

Objective

To familiar with bit manipulation methods.

Prelab

Read Instructions related to Logical and shift the operands.

Bit Manipulation instructions

| Instructions | Operands | Description |
| --- | --- | --- |
| AND | - REG, memory - memory, REG - REG, REG - memory,immediate - REG, immediate | Logical AND between all bits of two operands. Result is stored in operand1. These rules apply: 1 AND 1 = 1; 1 AND 0 = 0 0 AND 1 = 0; 0 AND 0 = 0 |
| OR | - REG, memory - memory, REG - REG, REG - memory, immediate - REG, immediate | Logical OR between all bits of two operands. Result is stored in first operand. These rules apply: 1 OR 1 = 1; 1 OR 0 = 1 0 OR 1 = 1; 0 OR 0 = 0 |
| XOR | - REG, memory - memory, REG - REG, REG - memory, immediate - REG, immediate | Logical XOR (Exclusive OR) between all bits of two operands. Result is stored in first operand. These rules apply: 1 XOR 1 = 0; 1 XOR 0 = 1 0 XOR 1 = 1; 0 XOR 0 = 0 |
| SHL | - memory, immediate - REG, immediate - memory, CL - REG, CL | Shift Left. Shift operand1 Left. The number of shifts is set by operand2. Algorithm: - Shift all bits left, the bit that goes off is set to CF. Zero bit is inserted to the right-most position. |
| SHR | - memory, immediate - REG, immediate - memory, CL - REG, CL | Shift Right. Shift operand1 Right. The number of shifts is set by operand2. Algorithm: - Shift all bits right, the bit that goes off is set to CF. - Zero bit is inserted to the left-most position. |
| ROL | - memory, immediate - REG, immediate - memory, CL - REG, CL | Rotate Left. Rotate operand1 left. The number of rotates is set by operand2. Algorithm: Shift all bits left, the bit that goes off is set to CF and the same bit is inserted to the right-most position. |
| RCL | - memory, immediate - REG, immediate - memory, CL - REG, CL | Rotate operand1 left through Carry Flag. The number of rotates is set by operand2. Algorithm: Shift all bits left, the bit that goes off is set to CF and previous value of CF is inserted to the right-most position. Example: STC ; set carry (CF=1). MOV AL, 1Ch ; AL = 00011100b RCL AL, 1 ; AL = 00111001b, CF=0. RET OF=0 if first operand keeps original sign. |
| ROR | - memory, immediate - REG, immediate - memory, CL - REG, CL | Rotate Right. Rotate operand1 right. The number of rotates is set by operand2. Algorithm: Shift all bits right, the bit that goes off is set to CF and the same bit is inserted to the left-most position. |

Exercises

7.1 Write a program to check if given data is positive or negative.

Ans. DATA SEGMENT

BLOCK1 DB -21H

MSG1 DB "POSITIVE$"

MSG2 DB "NEGATIVE$"

DATA ENDS

CODE SEGMENT

ASSUME CS:CODE,DS:DATA

START:  MOV AX,DATA

MOV DS,AX

MOV AL,BLOCK1

ROL AL,1

JC NEGA

LEA DX,MSG1

JMP LAST

NEGA:   LEA DX,MSG2

LAST:	INT 21H

CODE ENDS

END START

7.2 Write a program to check if given data is odd or even

Ans. DATA SEGMENT

BLOCK1 DB 21H

MSG1 DB "ODD$"

MSG2 DB "EVEN$"

DATA ENDS

CODE SEGMENT

ASSUME CS:CODE,DS:DATA

START:  MOV AX,DATA

MOV DS,AX

MOV AL,BLOCK1

ROR AL,1

JC ODD

LEA DX,MSG2

JMP LAST

ODD:    LEA DX,MSG1

LAST:   INT 21H

CODE ENDS

END START

7.3 Write a program to count the number of 1’s and 0’s in given data

Ans. DATA SEGMENT

BLOCK1 DW 1234H

TOTAL0 DB ?

TOTAL1 DB ?

DATA ENDS

CODE SEGMENT

ASSUME CS:CODE,DS:DATA

START:  MOV AX,DATA

MOV DS,AX

MOV CL,10H

MOV AX,BLOCK1

RE:	ROL AX,1

JC ONEE

INC BH

JMP LAST

ONEE:   INC BL

LAST:	DEC CL

JNZ RE

MOV TOTAL0,BH

MOV TOTAL1,BL

INT 21H

CODE ENDS

END START